# A5: Firewall

## 2. Set up the network

### # adding route on lab2 and lab3:

sudo ip route add 192.168.2.0/24 via 192.168.0.10 dev enp0s8

sudo ip route add 192.168.0.0/24 via 192.168.2.10 dev enp0s8

# enabling ipv4 forwarding on lab1:

sudo sysctl -w net.ipv4.conf.enp0s8.forwarding=1
sudo sysctl -w net.ipv4.conf.enp0s9.forwarding=1
sudo sysctl -w net.ipv4.conf.enp0s8.proxy_arp=1
sudo sysctl -w net.ipv4.conf.enp0s9.proxy_arp=1

sudo iptables -L # checking firewall rules

- Chains are associated with hooks

- Chains are triggered by hooks

- Chains determine when rules will be evaluated.

- hooks are implemented in the kernel level, each packet, whether ingress or egress, triggeres hooks

- There are five netfilter kernel hooks

- tables hold rules. Each family type e.g. ip, arp, nat, filter has separate table.

The iptables firewall works by interacting with the packet filtering hooks in the Linux kernel’s networking stack. These kernel hooks are known as the netfilter framework.

Every packet that passes through the networking layer (incoming or outgoing) will trigger these hooks, allowing programs to interact with the traffic at key points. The kernel modules associated with iptables register with these hooks in order to ensure that the traffic conforms to the conditions laid out by the firewall rules.

Reference:

Tables, chain, hook and rules

#### 3. Implement packet filtering on the router

On lab3:

sudo apt-get install proftpd apache2

On lab2:

sudo  apt install nmap

nmap -A -T4 192.168.2.11

With firewall:

On lab3:

On lab1:

sudo nft add table ip filter

# creating a base chain associated with FORWARD hook

sudo nft add chain ip filter FORWARD { type filter hook forward priority 0 \; }

# drop all forwarding packet by default

sudo nft add rule ip filter FORWARD counter drop

# finds rule with handle

nft -a list table filter

# deleting a rule

sudo nft delete rule filter FORWARD handle 5

# delete all the rules in a chain with the following command:

nft flush chain filter output

# delete all the rules in a table

nft flush table filter

# adding a rule at a given position

nft add rule filter output position 8 ip daddr 127.0.0.8 drop # adding this rule after handle 8

nft insert rule filter output position 8 ip daddr 127.0.0.8 drop # inserting this rule before handle 8

### # Allow ping from lab2

sudo nft add rule ip filter FORWARD iifname "enp0s8" oifname "enp0s9" ip protocol icmp icmp type echo-request counter accept

sudo nft add rule ip filter FORWARD iifname "enp0s9" oifname "enp0s8" ip protocol icmp icmp type echo-reply counter accept

# dropping and rejecting packet

sudo nft insert rule ip filter FORWARD position 3 iifname "enp0s8" oifname "enp0s9" ip protocol icmp icmp type echo-request counter reject

sudo nft insert rule ip filter FORWARD position 25 iifname "enp0s8" oifname "enp0s9" ip protocol icmp icmp type echo-request counter drop

### # Allow SSH from lab2 and to lab2

sudo nft insert rule ip filter FORWARD position 5 iifname "enp0s8" tcp dport 22 counter accept

sudo nft insert rule ip filter FORWARD position 5 oifname "enp0s8" tcp sport 22 counter accept

sudo nft insert rule ip filter FORWARD position 5 iifname "enp0s9" tcp dport 22 counter accept

sudo nft insert rule ip filter FORWARD position 5 oifname "enp0s9" tcp sport 22 counter accept

checking solution:

ssh vagrant@lab3  [from lab2]

ssh vagrant@lab2 [from lab3]

# Allowing http transfer

sudo nft insert rule ip filter FORWARD position 5 iifname "enp0s8" tcp dport {80, 443} counter accept

sudo nft insert rule ip filter FORWARD position 5 oifname "enp0s8" tcp sport {80, 443} counter accept

checking solution:

curl http://lab3

# Allowing ftp transfer

sudo nft insert rule ip filter FORWARD position 5 iifname "enp0s8" tcp dport {20, 21} counter accept

sudo nft insert rule ip filter FORWARD position 5 oifname "enp0s8" tcp sport {20, 21} counter accept

checking solution:

curl -o . ftp://vagrant:1234@lab3/home/vagrant/newfile

# Test cases to test the rules:

1. checking reachability of a machine by ping

2. SSH can be restricted and limited through firewall.

3. Limiting web access by specifying which port can be used. For example we can block http (80) traffic and can allow only https for our server.

4. File transfer via FTP.

5. With ping, we can also specify which way traffic is allowed. For example, lab2 can ping lab3 but not the other way unless we set a rule for that.

# all the ruleset

table ip filter {

chain FORWARD {

type filter hook forward priority filter; policy accept;

iifname "enp0s8" oifname "enp0s9" icmp type echo-request accept

iifname "enp0s9" oifname "enp0s8" icmp type echo-reply accept

iifname "enp0s8" tcp dport 22 counter packets 126 bytes 19212 accept

oifname "enp0s8" tcp sport 22 counter packets 145 bytes 24902 accept

iifname "enp0s9" tcp dport 22 counter packets 60 bytes 11411 accept

oifname "enp0s9" tcp sport 22 counter packets 81 bytes 14755 accept

iifname "enp0s8" tcp dport { 80, 443 } counter packets 218 bytes 16782 accept

oifname "enp0s8" tcp sport { 80, 443 } counter packets 181 bytes 115268 accept

iifname "enp0s8" tcp dport { 20, 21 } counter packets 109 bytes 6141 accept

oifname "enp0s8" tcp sport { 20, 21 } counter packets 95 bytes 7107 accept

counter packets 6343 bytes 344588 drop

}

}

## 4. Implement a web proxy

### # Capture HTTP Response Headers from lab3:

curl -I http://lab3

### # squid installation and configuration

sudo apt update

sudo apt install squid

sudo systemctl status squid

sudo nano “/etc/squid/squid.conf”

acl localnet src 192.168.0.11/24 # add this line 1333

http_access allow localnet # uncomment this line 1552

http_port 3128 transparent # 2107

sudo iptables -t nat -A PREROUTING -i enp0s8 -p tcp --dport 80 -j REDIRECT --to-port 3128

sudo iptables -t nat -A PREROUTING -i enp0s8 -s 192.168.1.0/24 -p tcp --dport 80 -j REDIRECT --to-port 3128

# adding firewall rule and forwarding port to 3128

nft add table ip nat

nft -- add chain ip nat prerouting { type nat hook prerouting priority -100 \; }

sudo nft add rule ip nat prerouting iifname "enp0s8" ip saddr 192.168.1.0/24 tcp dport 80 redirect to :3128

### # denying access to load lab3 webpage

acl lab3_web dst 192.168.2.11

http_access deny lab3_web

## 5. Implement a DMZ

# create a suitable table/chain setup for all further examples

add table nat

add chain nat prerouting { type nat hook prerouting priority dstnat; }

add chain nat postrouting { type nat hook postrouting priority srcnat; }

# redirect all traffic entering via eth0 to destination address 192.168.1.120

add rule nat prerouting iif eth0 dnat to 192.168.1.120

# translate source addresses of all packets leaving via eth0 to whatever locally generated packets would use as source to reach the same destination

add rule nat postrouting oif eth0 masquerade

# redirect incoming TCP traffic for port 22 to port 2222

add rule nat prerouting tcp dport 8080 redirect to :80

sudo nft add rule nat prerouting tcp dport 8080 redirect to :80